Environmental Pillar

Description and metrics of indicators

Indicator Structure and Theme ratings for the Environmental sustainability pillars

| PRODUCTION (En1) Scale: Low(-); Medium; High(+) |  |  |  |  |
| --- | --- | --- | --- | --- |
| THEME | SUB-THEME | Sub-sub theme | Sub-sub-sub-theme | INDICATOR |
| Abiotic Scale: Low(-); Medium; High(+) | Atmosphere emissions Scale: High(-);Medium; Low(+) |  |  | GHG Balance |
|  |  |  |  | Emission of Air Pollutants |
|  | Emission of Water Pollutants Scale: High(-);Medium; Low(+) |  |  | P balance |
|  |  |  |  | N balance |
|  | Resource depletion Scale: High(-);Medium; Low(+) | Resource use Scale: High(-);Medium; Low(+) | SynFertilisers Scale: High(-);Medium; Low(+) | SynP fertilisers |
|  |  |  |  | SynN fertilisers |
|  |  |  |  | Ground and Surface Water Withdrawals |
|  |  | Energy Efficiency Scale: Low(-); Medium; High(+) |  | SynN fertilisers |
|  |  |  |  | Diesel consumption |
| Ecosystem impact Scale: High(-);Medium; Low(+) |  |  |  | Land use |
|  |  |  |  | Land sharing/habitat provision |
|  |  |  |  | Soil Organic Matter |

| PROCESSING (En2) Scale: Low(-); Medium; High(+) |  |  |
| --- | --- | --- |
| THEME | SUB-TEME | INDICATOR |
| Energy Scale: High(-);Medium; Low(+) | Energy Efficiency Scale: Low(-); Medium; High(+) | Electricity |
|  |  | Other fuels |
|  | Renewable Energy % Scale: Low(-); Medium; High(+) | Renewable electricity |
|  |  | Renewable other sources |
|  | Atmosphere emissions Scale: High(-);Medium; Low(+) | GHG Balance |
|  |  | Emission of Air Pollutants |
| Waste generation Scale: High(-);Medium; Low(+) |  | Food Loss and Waste Reduction |
|  |  | Waste Disposal |
| Resource use Scale: High(-);Medium; Low(+) |  | Renewable and Recycled packaging |
|  |  | Ground and Surface Water Withdrawals |

| TRANSPORT AND DISTRIBUTION (En3) Scale: Low(-); Medium; High(+) |  |  |
| --- | --- | --- |
| THEME |  | INDICATOR |
| Environmental sustainability-Transportation Scale: Low(-); Medium; High(+) |  | Transport intensity |
|  |  | Atmosphere emissions |
|  |  | Loss of products |

| MARKETS AND RETAILERS (En4) Scale: Low(-); Medium; High(+) |  |  |
| --- | --- | --- |
| THEME |  | INDICATOR |
| Waste Scale: High(-);Medium; Low(+) |  | Food Loss and Waste Reduction |  |
|  |  | Waste Disposal |  |
| Energy Scale: High(-);Medium; Low(+) |  | Energy Efficiency |  |
|  |  | Renewable Energy % |  |
|  |  | Emission of Air Pollutants |  |
| Resource use Scale: High(-);Medium; Low(+) |  | Renewable and Recycled Materials |  |
|  |  | Packaging specification |  |

| CONSUMERS (En5) Scale: Low(-); Medium; High(+) |  |  |
| --- | --- | --- |
| THEME |  | INDICATOR |
| Waste Scale: High(-);Medium; Low(+) |  | Food Loss and Waste Reduction |  |
|  |  | Waste Disposal |  |
| Energy Scale: High(-);Medium; Low(+) |  | Cooking intensity |  |
|  |  | Renewable Energy % |  |
| Water use |  |  |  |

Table of Indicators for the Environmental sustainability pillars

| LEGEND: |  |
| --- | --- |
| En | Environmental Pillar for the Agri-food Chain |
| 1 | Production link |
| 2 | Processing link |
| 3 | Transport and Distribution link |
| 4 | Markets and Retailers link |
| 5 | Consumers link |

Table of Contents

DESCRIPTION AND METRICS OF ENVIRONMENTAL INDICATORS	6

GHG Balance	6

Emission of Air Pollutants	8

P balance	9

N balance	10

SynP fertilisers	11

SynN fertilisers	12

Ground and Surface Water Withdrawals	13

SynN fertilisers	14

Diesel consumption	15

Land use	16

Land sharing/habitat provision	17

Soil Organic Matter	18

Electricity	19

Other fuels	20

Renewable electricity	21

Renewable other sources	22

GHG Balance	23

Emission of Air Pollutants	24

Food Loss and Waste Reduction	25

Waste Disposal	26

Renewable and Recycled packaging	27

Ground and Surface Water Withdrawals	28

Transport intensity	29

Atmosphere emissions	30

Loss of products	31

Food Loss and Waste Reduction	32

Waste Disposal	33

Energy Efficiency	34

Renewable Energy %	35

Emission of Air Pollutants	36

Renewable and Recycled Materials	37

Packaging specification	38

Food Loss and Waste Reduction	39

Waste Disposal	40

Cooking intensity	41

Renewable Energy %	42

Water use	43

DESCRIPTION AND METRICS OF SOCIAL ThEMES AND SUBThEMES	44

PRODUCTION (En1)	44

En1.1 Abiotic	44

En1.1.1 Atmosphere emissions	44

En1.1.2 Emission of Water Pollutants	45

En1.1.3 Resource depletion	46

En1.1.3.1 Resource use	46

En1.1.3.1.1 Syn Fertilisers	47

En1.1.3.2 Energy Efficiency	47

En1.2 Ecosystem impact	48

PROCESSING (En2)	48

En2.1 Energy	48

En2.1.1 Energy Efficiency	49

En2.1.2 Renewable Energy %	49

En2.1.3 Atmosphere emissions	50

En2.2 Waste generation	50

En2.3 Resource use	51

TRANSPORT AND DISTRIBUTION (En3)	51

Transport and Distribution (En3)	51

MARKETS AND RETAILERS (En4)	52

En4.1 Waste	52

En4.2 Energy	52

En4.3 Resource use	53

CONSUMERS (En5)	53

En5.1 Waste	53

En5.2 Energy	54

Reference list	55

# DESCRIPTION AND METRICS OF ENVIRONMENTAL INDICATORS

INDICATOR NAME:

SUB-THEME:

THEME:

LINK:

## GHG Balance

Atmosphere emissions

Abiotic

Production (En1)

Crop production contributes to climate change via the emission of greenhouse gases (GHGs), primarily from use of synthetic nitrogen fertilisers (SNF) that require considerable amount of fossil energy to produce, and that give rise to soil emissions of nitrous oxide following application. Meanwhile, depletion or enhancement of soil carbon (C) stocks can results in significant emission to, or sequestration from, the atmospheric pool of carbon dioxide (CO2) that influences temperature forcing (and thus climate change).

The primary metric proposed for this environmental aspect of crop production is GHG emissions per tonne (Mg) of produce. GHG emissions intensities can be: (i) derived through application of a life cycle assessment (LCA) to specific crop cultivation stages, most simply by using an open source footprint calculator such as the Cool farm Tool (https://coolfarmtool.org/) or AgreCalc (https://www.agrecalc.com/); (ii) obtained from secondary sources, including LCA (e.g. Ecoinvent & AgriFootprint) databases and published food LCA studies (e.g. Poore and Nemecek 2018).

Results should be expressed per tonne (Mg) of dry matter (DM) of the main product, to ensure standardized units. This may require knowledge of moisture content at harvest, and possibly also allocation of crop cultivation burdens across multiple co-products (e.g. grain and straw) based on relative economic values. GHG emissions should be aggregated as kg CO2 equivalent based on IPCC (2013) global warming potentials over a 100-year timeframe (GWP100). Changes in soil C stocks may be included in net emissions balances, as per standard reporting guidelines for land use and land use change emissions (BSI 2011; IPCC 2006b).

Note that for this and all subsequent environment metrics, harvested products include co-products (e.g. straw) alongside the main product (e.g. grain) harvested from the cropping system.

The use of an appropriate carbon calculator such as Cool Farm Tool is the 'gold standard' approach to represent GHG emission performance in primary production as per the thresholds set out below, and is necessary for animal systems. However, for cropping systems, where it is not feasible to enter data into a carbon calculator, an alternative approach is to use nitrogen fertiliser application rate as a proxy of GHG emissions. Approximately 80% of crop emissions (up to the farm gate) originate from nitrogen fertiliser manufacture and nitrous oxide emissions that arise in proportion to nitrogen application rates. Thus, it is the single most relevant proxy for pre farm gate GHG emissions. The same thresholds are proposed for this metric as for the energy intensity of primary production indicator. The following scales are provisionally suggested based on range of crop footprint values (Wernet et al. 2016), and may need calibrating.

The following scales are provisionally suggested based on range of crop footprint values (Wernet et al. 2016), and may need calibrating.

Low:		Less than 250 kg CO2 eq. Mg-1 DM harvested product

Medium:	Between 250 and 500 kg CO2 eq. Mg-1 DM harvested product

High:		Over 500 kg CO2 eq. Mg-1 DM harvested product

INDICATOR NAME:

SUB-THEME:

THEME:

LINK:

Emission of Air Pollutants

Atmosphere emissions

Abiotic

Production (En1)

The application of SNF, especially in urea format, and organic fertilisers, also give rise to emissions of ammonia (NH3), which contributes to air pollution, human health and ecosystem damage. Through a reduction in requirements for SNF and organic fertilisers, legumes may be associated with lower atmospheric emissions than conventional crops.

Given the difficulty of obtaining precise data for the ideal metric of NH3 emission per Mg DM, the primary metric proposed for this environmental aspect of crop production is based on the intensity of activities giving rise to ammonia emissions – primarily mineral and organic fertilizer applications. Information on typical rates of fertilizer application (e.g. fertilizer recommendations) and yields for particular crops (e.g. EuroStat and FAO Stat) can provide a strong indication of the overall GHG- and ammonia- intensities of production. The application of urea and manure provide simpler metrics relating to the likely intensities of ammonia emissions.

Applications of SNF and organic fertilisers, in abated or unabated forms, expressed per Mg DM of harvested product (Misselbrook, TH; Gilhespy, SL; Cardenas, LM; Williams, J; Dragosits 2015; Webb and Misselbrook 2004).

Low:		No SNF, animal manures nor biogas digestate applied.

Medium:	SNF or organic N fertilisers applied, at rates less than 10 kg N Mg-1 DM in non-abated urea, manure or biogas digestate form.

High:		Over 10 kg N Mg-1 DM applied in non-abated urea, manure or biogas digestate form.

P balance

INDICATOR NAME:

SUB-THEME:

THEME:

LINK:

Emission of Water Pollutants

Abiotic

Production (En1)

Phosphorus (P) is a limiting nutrient in many freshwater systems. Loss of P from cropping systems into water courses is a major cause of eutrophication (excessive nutrient enrichment) and associated algal blooms and biodiversity loss in freshwater bodies. It is related to the application of P in mineral and organic fertilisers, and soil erosion. A useful indicator of risk of P loss to water is P surplus. Usually, this is expressed as kg surplus (kg P in outputs minus kg P in inputs) per hectare.

[kg ha-1 P in outputs minus kg P ha-1 in inputs)/ kg P ha-1 in inputs] x 100%. Determine and quantify all types of crops (by area and yield) and animals (by heads or places and performance) in the operation. Quantify all imports and exports of nutrient-containing materials, such as fertilizers, feed and agricultural produce. Using an established method and recognized standard values, calculate the nitrogen and phosphorus supply and demand of the operation. Correct the nitrogen balance for volatile and, if possible, for liquid losses (leaching). Rate the nitrogen and the phosphorus balance of the operation by comparing effective supply with demand (SAFA “Nutrient balance (E 5.1.2)”)

High (-): 	>20%

Medium: 	10-20%

Low (+): 	<10%

N balance

INDICATOR NAME:

SUB-THEME:

THEME:

LINK:

Emission of Water Pollutants

Abiotic

Production (En1)

Nitrogen (N) is a limiting nutrient in some freshwater and many coastal marine ecosystems. Leaching of N from cropping systems into water courses is a major cause of eutrophication (excessive nutrient enrichment) and associated algal blooms and biodiversity loss in freshwater and coastal marine water bodies. N leaching is closely related to total N application in SNF and organic fertilisers. A useful indicator of resource (in)efficiency and risk of N loss to water is N surplus. As for P surplus, we propose that N surplus is initially calculated as kg N outputs minus kg N inputs per hectare, then expressed as a percentage of inputs (see “Nutrient balance (E 5.1.2)” of the SAFA Indicator recommendations).

[kg ha-1 N in outputs minus kg N ha-1 in inputs)/ kg N ha-1 in inputs] x 100%. Determine and quantify all types of crops (by area and yield) and animals (by heads or places and performance) in the operation. Quantify all imports and exports of nutrient-containing materials, such as fertilizers, feed and agricultural produce. Using an established method and recognized standard values, calculate the nitrogen and phosphorus supply and demand of the operation. Correct the nitrogen balance for volatile and, if possible, for liquid losses (leaching). Rate the nitrogen and the phosphorus balance of the operation by comparing effective supply with demand. See SAFA “Nutrient balance (E 5.1.2)”

Scale: High(-); Medium; Low (+)

High (-): 	>20%

Medium: 	10-20%

Low (+): 	<10%

INDICATOR NAME:

SUB-SUB-SUB-THEME:

SUB-SUB-THEME:

SUB-THEME:

THEME:

LINK:

SynP fertilisers

Syn Fertilisers

Resource use

Resource depletion

Abiotic

Production (En1)

Mineral P fertilizer is produced from finite reserves of phosphate rock, proven reserves of which are forecast  to be depleted by the end of the century (Cordell, Drangert, and White 2009). High rates of mineral P fertilizer use is therefore a hotspot for abiotic resource depletion within crop production systems. Here, we propose the absolute rate of mineral P fertilizer application (excluding P in manures and biofertilzers) per Mg DM harvested product as a useful indicator of comparative resource depletion across crop production systems.

Kg mineral P fertilizer application per Mg DM harvested product. Examples of crop-specific nutrient requirements and recommended fertiliser application rates can be found in nutrient management guidelines, e.g. AHDB (2017).

Low: 		< 2 kg P Mg-1 DM harvested product

Medium: 	2-5 kg P Mg-1 DM harvested product

High:		> 5 kg P Mg-1 DM harvested product

INDICATOR NAME:

SUB-SUB-SUB-THEME:

SUB-SUB-THEME:

SUB-THEME:

THEME:

LINK:

SynN fertilisers

Syn Fertilisers

Resource use

Resource depletion

Abiotic

Production (En1)

Synthetic N fertilizer manufacture requires large amounts of energy, usefully in the form of natural gas or other finite fossil fuels. Fertilizer manufacture is a major driver of energy consumption in the life cycle of crop production. Here, we propose the absolute rate of SNF application (excluding N in manures and biofertilzers) per Mg DM harvested product as a useful indicator of comparative resource depletion across crop production systems.

Kg SNF applied per Mg DM harvested product.  Examples of crop-specific nutrient requirements and recommended fertiliser application rates can be found in nutrient management guidelines, e.g. AHDB (2017).

Low: 		< 5 kg N Mg-1 DM harvested product

Medium:	5-15 kg N Mg-1 DM harvested product

High: 		> 15 kg N Mg-1 DM harvested product

INDICATOR NAME:

SUB-SUB-THEME:

SUB-THEME:

THEME:

LINK:

Ground and Surface Water Withdrawals

Resource use

Resource depletion

Abiotic

Production (En1)

Globally, agriculture is responsible for over 70% of freshwater abstraction, often at rates in excess of natural recharge, leading to lowering of water tables and representing a fundamentally unsustainable practice. This challenge is likely to be exacerbated by climate change. However, data on water abstraction are often not collated, and here we propose a simple indicator of potential water stress based on irrigation practice. Where no irrigation is needed, water stress induced by cropping is assumed to be minor. Where irrigation is required, practices are differentiated into “advanced” methods that maximise water use efficiency, such as drip irrigation and deficit (control) irrigation, and less efficient (basic) irrigation methods such as flood irrigation and sprinkler irrigation.

Type of irrigation practice implemented for the crop (Antonopoulos et al. 2014).

High (-):	Sprinkler/flood irrigation employed

Medium: 	Advanced irrigation employed (e.g. drip irrigation, deficit irrigation, etc).

Low (+):	None

INDICATOR NAME:

SUB-SUB-THEME:

SUB-THEME:

THEME:

LINK:

SynN fertilisers

Energy Efficiency

Resource depletion

Abiotic

Production (En1)

Synthetic N fertilizer manufacture requires large amounts of energy, usefully in the form of natural gas or other finite fossil fuels. Fertilizer manufacture is a major driver of energy consumption in the life cycle of crop production. Here, we propose the absolute rate of SNF application (excluding N in manures and biofertilzers) per Mg DM harvested product as a useful indicator of comparative resource depletion across crop production systems.

Kg SNF applied per Mg DM harvested product. Examples of crop-specific nutrient requirements and recommended fertiliser application rates can be found in nutrient management guidelines, e.g. AHDB (2017).

Low: 		< 5 kg N Mg-1 DM harvested product

Medium:	5-15 kg N Mg-1 DM harvested product

High: 		> 15 kg N Mg-1 DM harvested product

Diesel consumption

INDICATOR NAME:

SUB-SUB-THEME:

SUB-THEME:

THEME:

LINK:

Energy Efficiency

Resource depletion

Abiotic

Production (En1)

The main on-farm use of energy in cropping systems is usually the combustion of diesel to power agricultural machinery. If fuel is used for e.g. on-farm grain drying, then this fuel consumption may be included in the proposed metric, which is simply the volume (L) of fossil fuel consumed per Mg DM harvested product. For the purposes of this simple metric, it is not necessary to differentiate by type of fossil fuel (petrol/diesel/liquid petroleum gas). However, biofuels should be excluded from the volumes.

Fuel consumption, L Mg-1 harvested product. The following scales are provisionally suggested based on range of crop footprint values (Wernet et al. 2016), and may need calibrating.

High (-): 	>18L Mg-1 DM harvested product

Medium: 	12-18L Mg-1 DM harvested product

Low (-): 	<12L Mg-1 DM harvested product

Land use

INDICATOR NAME:

THEME:

LINK:

Ecosystem impact

Production (En1)

Agriculture utilizes 70% of global land area. Land occupation by agriculture is a major driver of habitat loss and soil degradation. Here, we propose a simple metric based on the area required overt time to produce one Mg DM of harvested product. The metric is m2.yr. Crucially, if multiple crops are harvested in one year on a particular area of land, then this should be represented within the metric by dividing the area by the fraction of year allocated to the crop in question. For example, if two soybean crops are cultivated within one year in a particular region, then the yield-derived m2 Mg-1 required for each crop may be divided by 2 to produce the final specific time-weighted area (m2.yr Mg-1 DM). This metric provides an indication of land use efficiency and therefore could be used to identify “land sparing” opportunities, or pressures driving agricultural expansion that may ultimately lead to undesirable (indirect) land use change (Searchinger et al. 2018).

M2.yr Mg-1 DM harvested product. The following scales are provisionally suggested based on range of crop yields (FAO 2018), and may need calibrating.

High (-): 	> 2000 m2.yr Mg-1 DM harvested product

Medium: 	1000-2000 m2.yr Mg-1 DM harvested product

Low (+): 	< 1000 m2.yr Mg-1 DM harvested product

Land sharing/habitat provision

INDICATOR NAME:

THEME:

LINK:

Ecosystem impact

Production (En1)

Land occupation for agriculture, as measured in the preceding metric, provides an important indication of pressure on natural habitats. However, agriculturally utilized land, such as extensive livestock grazing on low-input pastures, can support high nature value (Haddaway, Styles, and Pullin 2014). This metric therefore represents the extent to which land utilized for agricultural purposes is able to deliver a wider suite of ecosystem services (outside of food provisioning) – i.e. “land sharing”.

Determine the total area of the ecosystems used in the operations and directly affected by these operations. Determine the share of land or aquatic and marine habitat, where the structural diversity of habitats – aquatic and terrestrial – is at least as high as in natural ecosystems of the region. Structural diversity pertains to the vertical layering and horizontal heterogeneity of habitats at the patch and landscape levels (SAFA indicator E4.1.3, 2013).

High (+):	Structural diversity on the complete utilized and adjacent land is at least as high as in natural ecosystems of the same region. Polyculture is practiced both in land and in aquatic (i.e. multitrophic) operations.

Medium:	Different areas of land affected by relevant operations can be categorized as both High and Low.

Low (-):	All utilized and adjacent land/aquatic habitat is covered by monocultures with a single habitat layer and no substantial horizontal heterogeneity, although the landscape would be structurally diverse without human influence.

Soil Organic Matter

INDICATOR NAME:

THEME:

LINK:

Ecosystem impact

Production (En1)

Soil organic matter (SOM) has been declining for decades on cropland soils across Europe, and has been proposed as a useful proxy indicator for soil quality (Mila, RomanyÃ, and Cowell 2007). SOM is closely related to biological activity, water retention and general fertility of cropland soils. However, SOM varies considerably depending on factors outside of farmer management practises, including climate and soil type. Here, we propose SOM concentration (FAO 2013) in the topsoil (0-10 cm) of fields producing the assessed crop as a metric of soil quality. This metric applies only to cropland soils, as grassland soils have higher SOM. Often, SOM is reported in terms of soil organic carbon (de Brogniez et al. 2015); typically, SOM is twice the mass of soil organic carbon (IPCC 2006a).

SOM concentration, expressed as a percentage weight of dried soil, in soil samples taken from top 10 cm  of topsoil.

Low (-): 	<2% SOM content in top 10 cm of soil.

Medium:	2-4% SOM content in top 10 cm of soil.

High (+):	>4% SOM content in top 10 cm of soil.

Electricity

INDICATOR NAME:

SUB-THEME:

THEME:

LINK:

Energy Efficiency

Energy

Processing (E2)

Typically, the main direct source of energy consumed in crop processing is electricity. Generation of electricity using fossil fuels gives rise to GHG emissions and air pollution, whilst depleting finite fossil fuel reserves. Electricity consumption is usually measured at site level for billing purposes, but may be monitored at a smaller scale (building or process scale) in some cases. The metric proposed here is simply the quantity of electricity used to process one tonne (Mg) DM product. For sites processing just one main crop/commodity, total site-level electricity consumption over a given time period can simply be divided by output of processed product over that same time period. For sites processing multiple commodities, site level electricity consumption may be allocated across products based on weight (unless more accurate splits are possible based on known intensities of processing). Similarly, electricity consumption may be allocated across co-products based on e.g. relative mass, gross energy value or economic value (Finkbeiner et al. 2006). Note that product output (rather than throughput) excludes waste streams arising from production.

Total electricity consumption, kWh Mg-1 DM product output (includes renewable electricity).

Scale: High (-); Medium; Low (+)

Other fuels

INDICATOR NAME:

SUB-THEME:

THEME:

LINK:

Energy Efficiency

Energy

Processing (E2)

In addition to electricity use, a considerable amount of energy may be required in the form of fuels for heating during crop/commodity processing – e.g. for drying, boiling, etc. A range of fuel types may be used, the most common being kerosene, natural gas and liquified petroleum gas (LPG). The efficiency metric here requires all fuels to be compared in terms of their lower heating value (LHV). The LHV of common fuel types can be found in (DEFRA 2019), and should be summed across all types of fuel consumed on site. Total fuel consumption, expressed as MJ LHV, may then be allocated to the main product output as described above for electricity consumption.

Total fuel consumption, MJ / Mg-1 DM product output (includes renewable fuels such as biogas and wood pellets).

Scale: High (-); Medium; Low (+)

Renewable electricity

INDICATOR NAME:

SUB-THEME:

THEME:

LINK:

Renewable Energy %

Energy

Processing (E2)

The previous metric on total electricity consumption provides an indication of energy efficiency. The environmental impact generated by that electricity consumption is heavily dependent upon the type of electricity used. Fossil fuel electricity drives large environmental impact, whilst electricity generated from renewable sources such as wind, solar photovoltaic and hydro- drives much lower environmental impact. This metric therefore assesses the sustainability of the specific electricity supply. In order to avoid double-counting of renewable electricity generation in the grid mix, only onsite or dedicated additional renewable electricity generation is considered here, as per carbon footprint guidelines (BSI 2011). Thus, the purchase of renewable electricity from the grid does not count as dedicated renewable electricity in this metric.

Percentage share of dedicated (non-grid) renewable electricity consumption in relation to total electricity consumption.

Share of dedicated renewable electricity consumption

Low (-): 	<20%

Medium: 	20-50%

High (+): 	>50%

Renewable other sources

INDICATOR NAME:

SUB-THEME:

THEME:

LINK:

Renewable Energy %

Energy

Processing (E2)

As with electricity use, it is important to determine what proportion of other fuel use originates from more sustainable, renewable sources. These may include biogas and wood pellets. The proportion of these sources in the fuel mix can be calculated as a percentage based on MJ of LHV (DEFRA 2019).

Percentage share of renewable fuels in relation to total (non-electricity) onsite fuel consumption.

Share of onsite fuel consumption that is renewable.

Low (-): 	<20%

Medium:	20-50%

High (+):	>50%

INDICATOR NAME:

SUB-THEME:

THEME:

LINK:

GHG Balance

Atmosphere emissions

Energy

Processing (En2)

The significant energy inputs required for processing often give rise to GHG emissions via use of fossil fuels as energy carriers. For example, the generation of electricity used in processing, or the direct combustion of natural gas or oil in boilers to generate heat onsite. These emissions are highly variable, depending on the source of energy and the quantities used, which depend on the type of food products being produced. In addition, leakage of refrigerants from cooling systems can contribute significantly to GHG emissions. Whilst the ideal metric for GHG emissions from processing would combine information on energy use and energy source to estimate total GHG emissions, these data are rarely available, and processing covers such as wide range of activities that it is impossible to propose universally applicable thresholds. Thus, a simplified approach is to use the main source of energy (accounting for than 50% of energy inputs) as a proxy for the GHG intensity of energy. Coal and oil are the most GHG-intensive energy sources per MJ, followed by gas, and then renewables as the least GHG-intensive sources of energy per MJ. The GHG intensity of electricity strongly depends on national grid profiles, but it reducing strongly across EU member states in line with various EU policies. Therefore, electricity is proposed as an intermediate GHG-intensive source of energy, unless it can be demonstrated that national grid average electricity generation in the relevant country where processing takes places is less than 0.3 kg CO2 eq. per kWh.

Grids with low GHG generation would include France, Sweden, Norway, possibly also UK now, and some others

High: Most energy comes from coal and oil

Medium: Most energy comes from natural gas or electricity

Low: Most energy comes from renewable (or electricity is from a national grid with low GHG intensity generation)

INDICATOR NAME:

SUB-THEME:

THEME:

LINK:

Emission of Air Pollutants

Atmosphere emissions

Energy

Processing (En2)

Combustion of fuels onsite for heat is the main source of polluting emissions to air during processing. Emissions of particular concern regarding human health and ecosystem damage include nitrogen oxides (NOx), volatile organic compounds (VOCs) and particulate matter (PM). Solid fuels including coal and wood, and oil, are responsible for much higher emissions of these species than natural gas. If no solid or fossil fuels are combusted, then onsite emissions will be negligible. Use of renewable sources of heat, or use of electricity for heating., can avoid onsite emissions, but may incur upstream emissions. These are almost always much lower owing to better combustion control and abatement technologies for large-scale power generation compared with smaller boilers. Given that these emissions cannot be easily measured in small-scale processing factories, a simple indicator based on the type of fuel combusted onsite is proposed.

Type of fuel combusted onsite for process heat.

High:		High-solid fuels or oil

Medium:	Gas

Low:		Non

Food Loss and Waste Reduction

INDICATOR NAME:

THEME:

LINK:

Waste generation

Processing (En2)

Loss of food through the processing value chain not only creates waste that needs to be managed, incurring environmental impact, but also generates wider environmental pressure by driving additional production to compensate for foregone useful output. A simple metric is proposed here, based on the percentage of commodity entering a processing chain, expressed on a dry matter basis, that exits the processing chain as (a) useful product(s).

Percentage of dry matter entering processing chain that leaves the chain as (a) product(s) for onward transport to distribution for consumption.

Low (-): 	<80% inputs as products

Medium:	80-90% inputs as products

High (+): 	>90% inputs as products

Waste Disposal

INDICATOR NAME:

THEME:

LINK:

Waste generation

Processing (En2)

Some waste is inevitably generated during processing, and the way in which this waste is managed determines the environmental pollution associated with it, and the overall resource efficiency of the value chain. Organic waste arising from the processed commodity may be reused or recycled according to the following waste hierarchy (most efficient option first): animal feed, anaerobic digestion, composting (Tufvesson, Lantz, and Börjesson 2013). Packaging and other waste may be reused or recycled via e.g. return of pallets for reuse, separation and diversion of plastics, metals, paper, glass from residual waste streams into recycling streams. This metric reflects the percentage of waste generated, by mass, that enters a reuse or recycling stream rather than the residual waste stream.

Percentage of total waste, by mass, that is separated and sent for reuse or recycling.

Percentage mass of total waste generated that is sent for reuse or recycling:

Low (-):		<50% recycling

Medium:	50-80% recycling

High (+): 	>80% recycling

INDICATOR NAME:

THEME:

LINK:

Renewable and Recycled packaging

Resource use

Processing (En2)

Packaging is a significant source of waste and non-renewable material use within food value chains. Crucially, it is a major source of single-use plastic that is increasingly recognized for its contribution to littering and ecosystem pollution (Dris et al. 2015). The previous metric addresses the management of waste arising in the processing chain. Packaging waste usually arises at the consumption stage of the value chain, but is heavily influenced by the actions of processors and retailers who package products and specific packaging requirements, respectively. Therefore, this metric places an onus on processors to use renewable (e.g. bioplastic: Álvarez-Chávez et al. 2012) and recycled materials in their packaging.  A subsequent metric, aimed at market actors who determine packaging specifications, relates to the quantity of packaging used.

Percentage of packaging material, by mass, that is recycled or renewable material.

Percentage of packing by weight that is either renewable (e.g. paper, bioplastics) or recycled.

Low (-): 	<50% renewable or recycled

Medium 	50-80% renewable or recycled

High (+):	>80% renewable or recycled

INDICATOR NAME:

THEME:

LINK:

Ground and Surface Water Withdrawals

Resource use

Processing (En2)

Processing foods involves use of water as an ingredient, for cooking, for rinsing produce and for general cleaning. Water may also be needed for cooling. The impact of water use is heavily dependent on where it is sourced from, whether or not it is returned to a nearby waterbody in a clean state, and how water stressed the local region is. An ideal indicator for water stress would be the Available Water Remaining method for water foot-printing (Boulay et al. 2018). However, this method involves a considerable amount of data and effort. Therefore, a simple metric of total water used for processing divided by the tonnes of product produced is proposed.

M3/tonne of product

High (-):

Medium:

Low (+):

Transport intensity

INDICATOR NAME:

THEME:

LINK:

Transport and Distribution

Transport and Distribution (En3)

Whilst there has been a lot of focus on “food miles” as a proxy for the environmental footprint of consumed food, it has been shown that transport typically accounts for a small share of the carbon footprint of many food products (Edwards-Jones et al. 2008). The distance travelled is therefore not a useful indicator of sustainability, given that e.g. tomatoes imported to the UK in winter have a lower environmental footprint than tomatoes grown in heated greenhouses in the UK (Antonopoulos et al. 2014). The mode of transport strongly influences environmental impact, with air freight generating up to 500 times more CO2 eq. per tonne.km travelled than ocean transport over long distances (DEFRA 2019). Therefore, we propose mode of transport, and local vs global transport, as two important components of environmental sustainability (local supply chains within Europe are likely to be subject to higher levels of environmental regulation than global supply chains). Where products comprise multiple ingredients, this metric relates to the most environmentally intensive mode across any of the ingredients accounting for more than 10% by mass of the final product(s).

Highest intensity scale and mode of transport applicable to any ingredient accounting for >10% by mass of final product(s)

Low (-): 		Global airfreight

Medium/low:		Global ocean

Medium/high:		Within EU transport

High (+):		Local (+)

Atmosphere emissions

INDICATOR NAME:

THEME:

LINK:

Transport and Distribution

Transport and Distribution (En3)

The mode of transport can have a very significant influence on GHG and related air pollutant emissions. GHG emissions from transport can be estimated based on the main mode(s) and distances of transport for each tonne of product (or preceding ingredient). The aforementioned databases for LCA (Wernet et al., 2016) and carbon factors for company reporting (DEFRA, 2019) contain information on CO2 intensities per tonne-km for various transport modes. Therefore, users can sum distances travelled by different transport modes multiplied by respective CO2 intensities.

Kg CO2e/t product dry matter (DM). Here is a link to the calculation tool (GHG Emissions from Transport or Mobile Sourcesand) and the DEFRA (2019)

High (-):		>100 kg CO2e/t DM

Medium: 		20-100 kg CO2e/t DM

Low (+):		<20kg CO2e/t DM

Loss of products

INDICATOR NAME:

THEME:

LINK:

Transport and Distribution

Transport and Distribution (En3)

A significant proportion of some perishable products may deteriorate during transport, leading to wastage. As with processing waste, this has downstream environmental implications in terms of waste management and upstream environmental implications in terms of additional (excess) production requirement. This metric represents the proportion of product that enters the transport and distribution stage of the value chain that is lost as waste – i.e. that is not successfully conveyed as product to the next (market) stage of the value chain.

Percentage, by weight, of product transported that is lost from the value chain as waste

Low (-):		>5% loss (-)

Medium:	2-5% loss

High (+):	<2% loss (+)

Food Loss and Waste Reduction

INDICATOR NAME:

THEME:

LINK:

Waste

Markets and Retailers (En4)

Loss of food through the retail and market stage of the value chain not only creates waste that needs to be managed, incurring environmental impact, but also generates wider environmental pressure by driving additional production to compensate for foregone useful output. A simple metric is proposed here, based on the percentage of food stuffs purchased by retailers, expressed on a dry matter basis, that does not reach the consumer (via sales or donation to food banks).

Percentage of food by weight purchased by the retailer that does not reach the consumer, but is instead disposed of as waste.

Low (-): 	>5% food disposed of as waste

Medium:	2-5% food disposed of as waste

High (+): 	<2% food disposed of as waste

Waste Disposal

INDICATOR NAME:

THEME:

LINK:

Waste

Markets and Retailers (En4)

The way in which retail and market waste is managed determines the environmental pollution associated with it, and the overall resource efficiency of the value chain. Organic waste may be reused or recycled according to the following waste hierarchy (most efficient option first): animal feed, anaerobic digestion, composting (Tufvesson, Lantz, and Börjesson 2013). Packaging and other waste may be reused or recycled via e.g. return of pallets for reuse, separation and diversion of plastics, metals, paper, glass from residual waste streams into recycling streams. This metric reflects the percentage of waste generated, by mass, that enters a reuse or recycling stream rather than the residual waste stream.

Percentage of waste, by mass, that is sent for reuse or recycling.

Percentage mass of total waste generated that is sent for reuse or recycling:

Low (-):		<50% recycling

Medium:	50-80% recycling

High (+): 	>80% recycling

Energy Efficiency

INDICATOR NAME:

THEME:

LINK:

Energy

Markets and Retailers (En4)

Wholesalers and retailers consume a considerable amount of energy across store lighting, heating ventilation and air conditioning and chilling or freezing food stuffs (Schoenberger, Galvez-Martos, and Styles 2013). Energy consumption for wholesale and retail of food stuffs is strongly related to the storage and display temperature, with substantial amounts of energy required for chilling and freezing (Galvez-Martos, Styles, and Schoenberger 2013). It can be difficult to isolate the specific energy consumption required for this purpose from other forms of onsite energy demand at storage and sales outlets. Therefore, this metric relates the intensity of energy demand to the type of storage and display.

Category of storage/display.

The type of food storage and display:

Low (-):		Frozen

Medium:	Refrigeration

High (+):	Ambient

Renewable Energy %

INDICATOR NAME:

THEME:

LINK:

Energy

Markets and Retailers (En4)

The main form of energy used in the retail sector is electricity (Galvez-Martos, Styles, and Schoenberger 2013). Therefore, the share of electricity generated from dedicated additional renewable sources is the principle metric proposed here, and follows the same calculation as described for Renewable Electricity in the processing stage.

Percentage of total electricity consumption that is met by electricity generated from dedicated additional renewable sources (e.g. onsite renewable sources not counted in the grid mix).

Scale:

Low: <20% (-)

Medium: 20-50%;

High: >50% (+)

INDICATOR NAME:

THEME:

LINK:

Emission of Air Pollutants

Energy

Markets and Retailers (En4)

Combustion of fuels onsite for heat is the main source of polluting emissions to air from the retail and market stage of the value chain. Emissions of particular concern regarding human health and ecosystem damage include nitrogen oxides (NOx), volatile organic compounds (VOCs) and particulate matter (PM). Solid fuels including coal and wood, and oil, are responsible for much higher emissions of these species than natural gas. If no solid or fossil fuels are combusted, then onsite emissions will be negligible. Use of renewable sources of heat, or use of electricity for heating., can avoid onsite emissions, but may incur upstream emissions. These are almost always much lower owing to better combustion control and abatement technologies for large-scale power generation compared with smaller boilers. Given that these emissions cannot be easily measured in small-scale processing factories, a simple indicator based on the type of fuel combusted onsite is proposed.

Main fuel types combusted onsite.

Low:		Oil, solid fuel combustion

Medium:	Gas combustion

High:		No on site combustion

INDICATOR NAME:

THEME:

LINK:

Renewable and Recycled Materials

Resource use

Markets and Retailers (En4)

Retailers have a strong influence on product and packaging specification. Therefore, the metric for renewable and recyclable materials used in packaging specified in the processing stage is also included here.

As per processing sector.

Scale:

Low: <50% renewable recycled (-)

Medium: 50-80% renewable recycled

High: >80% renewable recycled (+)

Packaging specification

INDICATOR NAME:

THEME:

LINK:

Resource use

Markets and Retailers (En4)

To reflect the strong influence of retailers on packaging specification during process and also final display, this metric assesses the amount of packaging on final marketed products. A simplified approach is taken where the number of layers of packaging on the displayed product represent packaging intensity. For example, fruit and vegetables displayed loose on shelves would be associated with no layers of packaging.

Number of layers of packaging of displayed products.

Low (-): 	More than one layer (-)

Medium:	One layer

High (+):	No packaging

Food Loss and Waste Reduction

INDICATOR NAME:

THEME:

LINK:

Waste

Consumer (En5)

In industrialized countries, the majority of the circa 40% of food that is wasted arises at the final consumer stage of the food value chain. The metric proposed here is a simple percentage of food, by weight, that is not consumed but ends up being discarded.

Percentage of food not consumed but discarded.

Low (-): 	>25% product wasted

Medium:	10-25% product wasted

High (+): 	<10% product wasted

Waste Disposal

INDICATOR NAME:

THEME:

LINK:

Waste

Consumer (En5)

Some waste is inevitably generated during processing, and the way in which this waste is managed determines the environmental pollution associated with it, and the overall resource efficiency of the value chain. Organic waste arising from the processed commodity may be reused or recycled according to the following waste hierarchy (most efficient option first): animal feed, anaerobic digestion, composting (Tufvesson, Lantz, and Börjesson 2013). Packaging and other waste may be reused or recycled via e.g. return of pallets for reuse, separation and diversion of plastics, metals, paper, glass from residual waste streams into recycling streams. This metric reflects the percentage of waste generated, by mass, that enters a reuse or recycling stream rather than the residual waste stream.

Percentage of waste, by mass, that is sent for reuse or recycling.

Percentage mass of total waste generated that is sent for reuse or recycling:

Low (-):		<50% waste recycled

Medium:	50-80% waste recycled

High (+): 	>80% waste recycled

Cooking intensity

INDICATOR NAME:

THEME:

LINK:

Energy

Consumers (En5)

Large quantities of energy may be consumed at the final step of food preparation, especially for cooking. Based on the cooking duration and energy intensities of different cooking methods (Hager and Morawicki 2013), we propose a metric based on the type of cooking (if any) required.

Type of cooking required.

Low (-):			Pot boiling

Medium-low: 		Oven baking

Medium-high:		Pan

High (+):		No cooking

Renewable Energy %

INDICATOR NAME:

THEME:

LINK:

Energy

Consumer (En5)

Consumers use electricity for cooling food and for cooking, alongside natural gas which is also commonly used for cooking. The share of electricity (or gas) generated from dedicated additional renewable sources is the principle metric proposed here. It is unlikely that consumers have a source of biogas, so the majority of renewable energy will be in the form of electricity. This could be sourced from e.g. onsite solar photovoltaic panels or micro wind turbines. As described for Renewable Electricity in the processing stage, the renewable electricity must be additional to that already installed and accounted for in the grid mix.

Percentage of energy used for food storage and preparation that originates from dedicated additional (e.g. onsite) renewable sources.

Scale:

Low: <20% (-) renewable

Medium: 20-50% renewable

High: >50% (+) renewable

INDICATOR NAME:

THEME:

LINK:

Water use

Water use

Consumers (En5)

Domestic and commercial kitchens are a significant source of potable water use. Water use during food consumption is driven by food preparation, cooking and cleaning cooking equipment and dishes (Styles, Schoenberger, and Galvez-Martos 2015). The main differentiating factor across different types of food will be food preparation and cooking. Therefore, this metric combines “hotspot” processes for those steps to generate rankings of performance.

Type of food preparation and cooking required.

Use of water for:

High (-): 	Cleaning /soaking and boiling

Medium: 	Cleaning / soaking or boiling

Low (+): 	No cleaning/soaking nor boiling required

# DESCRIPTION AND METRICS OF SOCIAL THEMES AND SUBTHEMES

## PRODUCTION (En1)

The environmental pillar of the Production node is separated into Abiotic effects and Ecosystem impacts.

En1.1 Abiotic

THEME:

NODE:

Production (En1)

Abiotic effects relate to the consumption of resources, such as fertilisers and fossil fuels, and associated emissions to the air and water. Thus, three sub-themes are classified below Abiotic effects: Atmosphere emissions; Emissions of water pollutants; Resource depletion.

Scale: Low (-); Medium; High (+)

En1.1.1 Atmosphere emissions

SUB-THEME:

THEME:

NODE:

Abiotic (En1.1)

Production (En1)

Within the Abiotic theme, the Atmosphere emissions subtheme comprises the major emissions to air that drive two major environmental impacts: climate change, represented by the GHG balance indicator, and air pollution, represented by the Emissions of air pollutants indicator. The GHG balance can represent carbon offset via sequestration, whilst the Emissions of air pollutants is dominated by a few gases but represents a wide range of associated human health and ecosystem damage impacts.

Scale: High (-); Medium; Low (+)

En1.1.2 Emission of Water Pollutants

SUB-THEME:

THEME:

NODE:

Abiotic (En1.1)

Production (En1)

The Emissions of water pollutants theme represents the primary drivers of environmental water quality impact associated with agriculture, namely losses of nutrients that cause nutrient enrichment (eutrophication) and associated loss of biodiversity within water bodies. This subtheme is represented by two indicators: P balance and N Balance. Although not directly representing losses to water, they are easy to calculate metrics that correlate strongly with risk of nutrient loss to water.

Scale: High (-); Medium; Low (+)

En1.1.3 Resource depletion

SUB-THEME:

THEME:

NODE:

Abiotic (En1.1)

Production (En1)

The Resource depletion subtheme within the Abiotic theme represents the primary non-renewable inputs to agricultural production systems, primarily fertilisers and fossil energy. This subtheme is represented by two further subthemes: Resource use and Energy efficiency.

Scale: High (-); Medium; Low (+)

SUB-SUB-THEME:

SUB-THEME:

THEME:

NODE:

En1.1.3.1 Resource use

En1.1.3 Resource depletion

Abiotic (En1.1)

Production (En1)

The Resource use subtheme within the Resource depletion subtheme represents the non-energy inputs to agricultural production, primarily fertilisers and irrigation water. Although water is a renewable resource, it is often extracted from groundwater at rates in exceedance of natural recharge rates; therefore, use of groundwater for irrigation can represent unsustainable use of this renewable resource. Resource use is represented by the Synthetic fertilisers subtheme, and the Ground and Surface water withdrawals indicator.

Scale: High (-); Medium; Low (+)

SUB-SUB-SUB-THEME:

SUB-SUB-THEME:

SUB-THEME:

THEME:

NODE:

En1.1.3.1.1 Syn Fertilisers

En1.1.3.1 Resource use

En1.1.3 Resource depletion

Abiotic (En1.1)

Production (En1)

The Synthetic fertilisers sub-sub-sub theme represents use of finite resources and fossil energy to produce non-renewable fertilisers, including N fertilisers produced using large amounts of fossil fuel and P fertilisers produced from finite phosphate rock. This subtheme has two simple indicators: Synthetic P fertilisers and Synthetic N fertilisers.

Scale: High (-); Medium; Low (+)

SUB-SUB-THEME:

SUB-THEME:

THEME:

NODE:

En1.1.3.2 Energy Efficiency

En1.1.3 Resource depletion

Abiotic (En1.1)

Production (En1)

The Energy efficiency subtheme with the Resource depletion subtheme represents the main measurable processes responsible for most energy consumption within agricultural production, both directly and indirectly. The subtheme is represented by two indicators: Synthetic N fertilisers and Diesel consumption. The Synthetic N fertilisers indicators is repeated from the Synthetic fertilisers subtheme, because it represents multiple environmental impacts. In this case, synthetic N fertiliser is the major (indirect) driver of fossil energy consumption in cropping systems.

Scale: Low (-); Medium; High (+)

En1.2 Ecosystem impact

THEME:

NODE:

Production (En1)

Ecosystem impacts relate to wider land management and land degradation associated with agricultural production. These effects are less easy to quantify than Abiotic depletion, and are represented by three broad indicators: land use; Land sharing/habitat provision; Soil organic matter.

Scale: High (-); Medium; Low (+)

## PROCESSING (En2)

The main environmental themes within the Processing node are Energy, Waste generation and Resource use.

En2.1 Energy

THEME:

NODE:

Processing (En2)

Energy use is a major driver of environmental impact from food processing operations. This theme represents the amount of energy used, the source of that energy, and the air emissions impacts associated with the energy. Energy use is represented by three subthemes: Energy efficiency; Renewable energy; Atmosphere emissions.

Scale: High (-); Medium; Low (+)

En2.1.1

SUB-THEME:

THEME:

NODE:

Energy Efficiency

Energy (En2.1)

Processing (E2)

Within the Energy theme, the Energy efficiency subtheme represents the amount of energy that is required to process food products. It is represented by two indicators for the major types of energy consumed: Electricity, and Other fuels.

Scale: Low (-); Medium; High (+)

En2.1.2 Renewable Energy %

SUB-THEME:

THEME:

NODE:

Energy (En2.1)

Processing (E2)

The Renewable energy subtheme quantifies the share of total energy consumption that is provided from additional renewable resources. Thus, after being assessed on their efficiency in the use of energy, processors are then assessed in relation to how sustainably that energy is produced. As with the Energy subtheme, indicators are divided by energy source: Electricity, and Other fuels.

Scale: Low (-); Medium; High (+)

En2.1.

SUB-THEME:

THEME:

NODE:

3 Atmosphere emissions

Energy (En2.1)

Processing (E2)

Energy use is a major driver of GHG and air pollutant emissions. The Atmosphere emissions subtheme therefore represents the contribution of energy use to emissions that drive climate change and, via air pollution, health and ecosystem damage. This subtheme is represented by two indicators: GHG balance, and Emission of air pollutants.

Scale: High (-); Medium; Low (+)

En2.2 Waste generation

THEME:

NODE:

Processing (En2)

Waste generation is another important environmental aspect of food processing. A significant share of commodity ingredients, and associated packaging, may be disposed of during processing. This subtheme is represented by two indicators that reflect the amount of waste generated, and the management of that waste: Food loss and waste reduction, and Waste disposal.

Scale: High (-); Medium; Low (+)

En2.3 Resource use

THEME:

NODE:

Processing (En2)

Resource use relates to the use of non-energy resources captured under the Energy subtheme. For processing, these mainly comprise non-renewable packaging materials and water used for processing and cleaning operations. This subtheme is represented by the following two indicators: Renewable and recycled packaging, and Ground and surface water withdrawals.

Scale: High (-); Medium; Low (+)

## TRANSPORT AND DISTRIBUTION (En3)

There are no main environmental themes within the Transport and Distribution node.

THEME:

NODE:

## Transport and Distribution (En3)

Transport and Distribution (En3)

Scale: Low (-); Medium; High (+)

## MARKETS AND RETAILERS (En4)

As with the processing node, environmental aspects pertinent to the markets & retailers node are covered by three themes: Waste, Energy & Resource use.

En4.1 Waste

THEME:

NODE:

Markets and Retailers (En4)

The Waste theme represents the proportion of food purchased by retailed that does not end up with consumers, and the management of that waste. It is represented by two indicators: Food loss and waste reduction, and Waste disposal.

Scale: High (-); Medium; Low (+)

En4.2

THEME:

NODE:

Energy

Markets and Retailers (En4)

The Energy theme represents the main environmental aspects that can be readily documented and over which retailers have a high degree of control in relation to the source of energy and the associated emissions to air. Thus, this theme is represented by two indicators: Renewable energy, and Emission of air pollutants.

Scale: High (-); Medium; Low (+)

En4.

THEME:

NODE:

3 Resource use

Markets and Retailers (En4)

The Resource use theme represents the aspects of resource use over which retailers have direct control, outside of energy resources which are represented in the Energy theme. This theme primarily addresses resources used by retailers, or specified by retailers, for packaging. The theme is represented by two indicators that reflect the pathways of influence that retailers have on direct operations and via specifications provided to suppliers: Renewable and recycled materials, and packaging specification.

Scale: High (-); Medium; Low (+)

## CONSUMERS (En5)

Consumers at the end of the value chain play an important role in driving demand, generating waste and using energy and water during food storage and preparation. This node is therefore represented by two themes, Waste and Energy, and one direct indicator, Water use.

En5.1 Waste

THEME:

NODE:

Consumers (En5)

The Waste theme represents first the proportion of purchased food that is not consumed, but instead discarded, and secondly the management of that discarded food waste. This theme indicates pressure on food production, through demand increases linked with waste, in addition to pressure on waste management systems. It is therefore an important theme within the overall value chain, and is represented by two indicators: Food loss and waste reduction, and Waste disposal.

Scale: High (-); Medium; Low (+)

En5.2

THEME:

NODE:

Energy

Consumers (En5)

The Energy theme is less important than the waste theme in terms of overall value chain environmental impact, and is more difficult to quantify owing to lack of submetering (smart meters) of energy consumption in most homes. However, it is represented by two indicators that are easy to quantify and that give a good indication of the amount of energy required, and the environmental intensity of the source of that energy: Cooking intensity, and Renewable energy %.

Scale: High (-); Medium; Low (+)

# Reference list

AHDB. 2017. Nutrient Management Guide (RB209). Stoneleigh. https://ahdb.org.uk/projects/RB209.aspx (August 11, 2017).

Álvarez-Chávez, Clara Rosalía, Sally Edwards, Rafael Moure-Eraso, and Kenneth Geiser. 2012. “Sustainability of Bio-Based Plastics: General Comparative Analysis and Recommendations for Improvement.” Journal of Cleaner Production 23(1): 47–56. https://www.sciencedirect.com/science/article/pii/S095965261100374X (November 3, 2018).

Antonopoulos, Ioannis-Sofoklis et al. 2014. Best Environmental Management Practice for the Agriculture Sector-Crop and Animal Production Final Draft. https://susproc.jrc.ec.europa.eu/activities/emas/documents/AgricultureBEMP.pdf (August 21, 2019).

Boulay, Anne-Marie et al. 2018. “The WULCA Consensus Characterization Model for Water Scarcity Footprints: Assessing Impacts of Water Consumption Based on Available Water Remaining (AWARE).” The International Journal of Life Cycle Assessment 23(2): 368–78. http://link.springer.com/10.1007/s11367-017-1333-8 (November 3, 2018).

de Brogniez, D. et al. 2015. “A Map of the Topsoil Organic Carbon Content of Europe Generated by a Generalized Additive Model.” European Journal of Soil Science 66(1): 121–34. http://doi.wiley.com/10.1111/ejss.12193 (August 21, 2019).

BSI. 2011. PAS 2050:2011 Specification for the Assessment of the Life Cycle Greenhouse Gas Emissions of Goods and Services. http://shop.bsigroup.com/upload/shop/download/pas/pas2050.pdf (October 20, 2016).

Cordell, Dana, Jan-Olof Drangert, and Stuart White. 2009. “The Story of Phosphorus: Global Food Security and Food for Thought.” Global Environmental Change 19(2): 292–305. https://ezproxy.bangor.ac.uk/login?url=http://www.sciencedirect.com/science/article/pii/S095937800800099X.

DEFRA. 2019. “Greenhouse Gas Reporting: Conversion Factors 2019 - GOV.UK.” https://www.gov.uk/government/publications/greenhouse-gas-reporting-conversion-factors-2019 (August 21, 2019).

Dris, Rachid et al. 2015. “Beyond the Ocean: Contamination of Freshwater Ecosystems with (Micro-)Plastic Particles.” Environmental Chemistry 12(5): 539. http://www.publish.csiro.au/?paper=EN14172 (February 22, 2019).

Edwards-Jones, Gareth et al. 2008. “Testing the Assertion That â€˜local Food Is Bestâ€TM: The Challenges of an Evidence-Based Approach.” Trends in Food Science & Technology 19(5): 265–74. https://ezproxy.bangor.ac.uk/login?url=http://www.sciencedirect.com/science/article/pii/S0924224408000034.

FAO. 2018. “FAOSTAT Data.” http://www.fao.org/faostat/en/#data (May 23, 2018).

FAO, UN. 2013. SAFA: Sustainability Assessment of Food and Agriculture Systems Indicators. Rome. www.fao.org/publications (August 21, 2019).

Finkbeiner, Matthias et al. 2006. “The New International Standards for Life Cycle Assessment: ISO 14040 and ISO 14044.” Int J LCA 11(112): 80–85.

Galvez-Martos, J.-L., D. Styles, and H. Schoenberger. 2013. “Identified Best Environmental Management Practices to Improve the Energy Performance of the Retail Trade Sector in Europe.” Energy Policy 63.

Haddaway, N.R., D. Styles, and A.S. Pullin. 2014. “Evidence on the Environmental Impacts of Farm Land Abandonment in High Altitude/Mountain Regions: A Systematic Map.” Environmental Evidence 3(1).

Hager, Tiffany J., and Ruben Morawicki. 2013. “Energy Consumption during Cooking in the Residential Sector of Developed Nations: A Review.” Food Policy 40: 54–63. https://www.sciencedirect.com/science/article/pii/S0306919213000201?via%3Dihub (August 26, 2019).

IPCC. 2006a. 2006 IPCC Guidelines for National Greenhouse Gas Inventories Volume 4 Chapter 11. http://www.ipcc-nggip.iges.or.jp/public/2006gl/pdf/4_Volume4/V4_11_Ch11_N2O&CO2.pdf (October 20, 2016).

———. 2006b. 2006 IPCC Guidelines for National Greenhouse Gas Inventories Volume 4 Chapter 4. http://www.ipcc-nggip.iges.or.jp/public/2006gl/pdf/4_Volume4/V4_04_Ch4_Forest_Land.pdf (October 20, 2016).

MilÃ, i. Canals, Joan RomanyÃ, and Sarah J Cowell. 2007. “Method for Assessing Impacts on Life Support Functions (LSF) Related to the Use of âÂ€Â˜fertile LandâÂ€ÂTM in Life Cycle Assessment (LCA).” Journal of Cleaner Production 15(15): 1426–40.

Misselbrook, TH; Gilhespy, SL; Cardenas, LM; Williams, J; Dragosits, U. 2015. Inventory of Ammonia Emissions from UK Agriculture 2014 Inventory of Ammonia Emissions from UK Agriculture – 2014. https://uk-air.defra.gov.uk/assets/documents/reports/cat07/1605231002_nh3inv2014_Final_20112015.pdf (March 8, 2017).

Poore, J, and T Nemecek. 2018. “Reducing Food’s Environmental Impacts through Producers and Consumers.” Science (New York, N.Y.) 360(6392): 987–92. http://www.ncbi.nlm.nih.gov/pubmed/29853680 (January 11, 2019).

Schoenberger, Harald, Jose Luis Galvez-Martose, and David Styles. 2013. Best Environmental Management Practice in the Retail Trade Sector - RetailTradeSector.Pdf. Sevilla. http://susproc.jrc.ec.europa.eu/activities/emas/documents/RetailTradeSector.pdf.

Searchinger, Timothy D., Stefan Wirsenius, Tim Beringer, and Patrice Dumas. 2018. “Assessing the Efficiency of Changes in Land Use for Mitigating Climate Change.” Nature 564(7735): 249–53. http://www.nature.com/articles/s41586-018-0757-z (December 14, 2018).

Styles, D., E.M. Dominguez, and D. Chadwick. 2016. “Environmental Balance of the of the UK Biogas Sector: An Evaluation by Consequential Life Cycle Assessment.” Science of the Total Environment 560–561: 241–53.

Styles, D., H. Schoenberger, and J.L. Galvez-Martos. 2015. “Water Management in the European Hospitality Sector: Best Practice, Performance Benchmarks and Improvement Potential.” Tourism Management 46.

Tufvesson, Linda M., Mikael Lantz, and Pål Börjesson. 2013. “Environmental Performance of Biogas Produced from Industrial Residues Including Competition with Animal Feed – Life-Cycle Calculations According to Different Methodologies and Standards.” Journal of Cleaner Production 53: 214–23. https://www.sciencedirect.com/science/article/pii/S0959652613002060 (October 30, 2018).

Webb, J, and T H Misselbrook. 2004. “A Mass-Flow Model of Ammonia Emissions from UK Livestock Production.” Atmospheric Environment 38(14): 2163–76. https://ezproxy.bangor.ac.uk/login?url=http://www.sciencedirect.com/science/article/pii/S1352231004000950.

Wernet, Gregor et al. 2016. “The Ecoinvent Database Version 3 (Part I): Overview and Methodology.” The International Journal of Life Cycle Assessment 21(9): 1218–30. http://link.springer.com/10.1007/s11367-016-1087-8 (October 18, 2016).

Latest version of the Environmental Factsheets: August 2021

Please cite this report as follows:

Debeljak, M., Dergan, T., Squire, G., Centofanti, T., Williams, M., Saget, S., Styles, D., Costa, M., Shrestha, S., Toma, L., Faical, A., Iannetta, P., Balázs, B., Keleman, E., Trajanov, A. (2020). Decision support models: environment, economy, policy of legume systems for the EU-H2020 funded project, ‘TRansition paths to sUstainable legume-based systems in Europe’ (TRUE), under Grant Agreement Number 727973. Available online at: www.true-project.eu.